Microelectronics Systems

Lab Manual

| Student’s Chinese Name | [Student Name] |
| --- | --- |
| Student’s English Name | [Student Name] |
| Student’s UESTC ID# | [Student ID] |
| Student’s UoG ID# | [Student ID] |

General Instructions

Here are some general instructions for Microelectronics Systems lab:

- All students are advised to bring their lab manuals with them for each lab session. The lab manuals are supposed to be hard bound.

- Students are required to complete the questionnaires at the end of each lab exercise.

- A minimum of 7 out of 9 lab exercises must be completed to avoid CW.

- The lab manuals must be submitted in the week following the third lab session.

- All students must complete and sign the Declaration of Originality Form before submission of the lab manual.

| Declaration of Originality Form |  |  |
| --- | --- | --- |
| This form must be completed and signed and submitted with all assignments. |  |  |
| Please complete the information below (using BLOCK CAPITALS). |  |  |
| Name [Student Name] Student Number [Student ID] Course Name UESTCHN1006: Microelectronic Systems (Hainan) Assignment Number/Name Lab Report Submission |  |  |
| An extract from the University’s Statement on Plagiarism is provided overleaf. Please read carefully THEN read and sign the declaration below. |  |  |
| I confirm that this assignment is my own work and that I have: |  |  |
| Read and understood the guidance on plagiarism in the Student Handbook, including the University of Glasgow Statement on Plagiarism |  |  |
| Clearly referenced, in both the text and the bibliography or references, all sources used in the work |  |  |
| Fully referenced (including page numbers) and used inverted commas for all text quoted from books, journals, web etc. (Please check with the Department which referencing style is to be used) |  |  |
| Provided the sources for all tables, figures, data etc. that are not my own work |  |  |
| Not made use of the work of any other student(s) past or present without acknowledgement. This includes any of my own work, that has been previously, or concurrently, submitted for assessment, either at this or any other educational institution, including school (see overleaf at 31.2) |  |  |
| Not sought or used the services of any professional agencies to produce this work |  |  |
| In addition, I understand that any false claim in respect of this work will result in disciplinary action in accordance with University regulations |  |  |
| DECLARATION: I am aware of and understand the University’s policy on plagiarism and I certify that this assignment is my own work, except where indicated by referencing, and that I have followed the good academic practices noted above Signed |  |  |

The University of Glasgow Plagiarism Statement

The following is an extract from the University of Glasgow Plagiarism Statement. The full statement can be found in the University Calendar at http://www.gla.ac.uk/media/media_413985_en.pdf#page=53&view=fitH,225. This should be read in conjunction with the discipline specific guidance provided by the School.

31.1 The University's degrees and other academic awards are given in recognition of a student's personal achievement. All work submitted by students for assessment is accepted on the understanding that it is the student's own effort.

31.2 Plagiarism is defined as the submission or presentation of work, in any form, which is not one's own, without acknowledgement of the sources. Plagiarism includes inappropriate collaboration with others. Special cases of plagiarism can arise from a student using his or her own previous work (termed auto-plagiarism or self-plagiarism). Auto-plagiarism includes using work that has already been submitted for assessment at this University or for any other academic award.

31.3 The incorporation of material without formal and proper acknowledgement (even with no deliberate intent to cheat) can constitute plagiarism.

Work may be considered to be plagiarised if it consists of: a direct quotation; a close paraphrase; an unacknowledged summary of a source; direct copying or transcription. With regard to essays, reports and dissertations, the rule is: if information or ideas are obtained from any source, that source must be acknowledged according to the appropriate convention in that discipline; and any direct quotation must be placed in quotation marks and the source cited immediately. Any failure to acknowledge adequately or to cite properly other sources in submitted work is plagiarism. Under examination conditions, material learnt by rote or close paraphrase will be expected to follow the usual rules of reference citation otherwise it will be considered as plagiarism. Departments should provide guidance on other appropriate use of references in examination conditions.

31.4 Plagiarism is considered to be an act of fraudulence and an offence against University discipline. Alleged plagiarism, at whatever stage of a student's studies, whether before or after graduation, will be investigated and dealt with appropriately by the University.

31.5 The University reserves the right to use plagiarism detection systems, which may be externally based, in the interests of improving academic standards when assessing student work.

If you are still unsure or unclear about what plagiarism is or need advice on how to avoid it,

SEEK HELP NOW! You can contact any one of the following for assistance:

Lecturer 
Course Leader
Dissertation Supervisor
Adviser of Studies
Student Learning Service

Introduction to the Microcontroller

Objectives

After completing these laboratories, you should be able to programme an embedded microcontroller to:

- generate digital output signals

- receive and process digital input signals

- generate analogue output signals

- receive and process analogue input signals

- generate pulse width modulated signals

This set of experiments will be assessed by milestones. You need to get through all the milestones, as the project in this course requires you to be proficient in getting analogue and digital signals into and out of the microcontroller.

Overview - the microcontroller

The set of experiments is based around the mbed microcontroller development board. Each student will be given an mbed, which hopefully you will come to cherish and use widely throughout your years of study at GC-UESTC. This means that you need to keep your mbed for several courses so treat it well and protect it. You will also be given a small breadboard to enable you to easily connect components to your beloved mbed.

The mbed microcontroller development board is shown in Figure 1 below.

Figure 1 - Nucleo microcontroller development board and some details of the pins on the board

The development board is based on the Nucleo L432KC microcontroller, with a 32-bit ARM Cortex-M4 core running at 80 MHz (ARM processors are used in many contemporary products including iphones for example). The development board has many interfaces which are shown in Figure 2:

Figure 2 - Pins Legend

The real beauty of the mbed development board is the simplicity with which the microcontroller can be programmed. It comes with an extensive application programming interface (API), which is a large set of building blocks (libraries) that are effectively C++ utilities.  The API allows programs to be easily and quickly developed.

Code can be generated using a web-based complier; this means you can develop programs whenever you have internet access. Once compiled, programs are easily downloaded to the mbed via a USB connector, which is also used to power the mbed.

The connections from the board that you will use are power (3v3 and GND) and pins labelled as D or A. For example, the development board has many interfaces:

pins D0 to D6, D9 to D13 and A0 to A3 and A6 to A7 can be used as digital in and digital out.
pins A0 to A6 and D3 can also be configured for analogue input.
pins A3 and A4 can be configured for analogue output.

pins D0, D1, D9, D10, A2, A5 can also be configured for pulse width modulated output. Various other pins can be configured for serial communication such as CAN, SPI, I2C and UART connection.

Potential irregularities:

Pins A4 and A5 are only ADC input
Pins D7 and D8 are not used.
The pins marked PWMx/yN are inverted waveforms of the PWMx/y outputs.
For PWM, the x represents a timer and y is channel, so if you want a different frequency, you must use a different timer, i.e., PWM1 and PWM16. If you are happy with the same frequency but want different duty cycles, you can use PWM1/1, PWM1/2 etc.

There are 3 LEDs on the board, one for ‘power on’ (red LD2 next to pin D12), a tricolour LED, LD1 (red/green/orange) for status, and a green LD3 that is connected to pin D13 digital output. LD3 can be driven to test basic functions without the need for any external component connection. LD3 will be called LED1 in the programs. However just using a microcontroller for one LED would be pretty boring, so we will want to attach other stuff.

To connect external components to the microcontroller, you will need to plug the Nucleo board into a breadboard. You should consider which way round it should go and make sure the USB lead does not take valuable space.

It is useful first step to connect GND and the 3V3 to the power rails of the breadboard since they are used a lot in circuits.

You should be aware that various input and output pins have maximum voltage and current limits, which should not be exceeded, otherwise your cherished microcontroller will be terminally damaged. Microcontroller input pins should not see voltages in excess of 5V.

As you can see from Figure 1, the mbed has a number of voltage pins, including:
GND : the reference potential = 0V. This should always be used as the ground for your logic circuits.
VIN : 7V - 12V input if running on a battery rather than the USB
5V : 5.0 V USB output that comes directly from your USB
3V3 : 3.3 V regulated output from the on board regulator – this is the voltage referred to as VDD in the course notes, and should always be used as the +ve for your logic circuits.

- Getting Started

Figure 3.  Top-view of the mbed

mbed Account

First, you need to make an mbed account. Go to the mbed website, www.mbed.org, and follow the instructions on getting started (http://mbed.org/handbook/Setup-guide). While you make an account, you need to have your mbed connected to your computer via a USB cable.

- Download a Program

Once you have created your mbed account, you can download your first program from the Getting Started web page - the usual starting place is the "Hello World" program. Save the program on your computer - perhaps create an mbed directory in your mapped drive, then copy the program to the mbed. This is where the mbed is really neat, as it just appears as a USB drive on your computer.

While the program is downloading to the mbed, the status LED (see Figure 3) on the mbed should flash. You may have to press the reset button on the mbed to get the program to run, but otherwise, one of the LEDs on the mbed should start to flash.

- Chose the chip type, Create and Run a New Program

The default chip type may not be the same as the one we used in these exercises, so we should choose the right one for preparation. Firstly, click and open the device selecting page like Figure 4:

Figure 4 – Device selecting page

Then click the “Add Board” button and choose NUCLEO-L432KC as the platform used in compiler in STMicroelectronic label.

Figure 5 – Select L432KC in STMicroelectronic label

You use the web based compiler to create a program, simplest way will be to follow the instructions to Create A Program from the Getting Started web page of the mbed site. The compiler screen looks like:

Figure 6 - Screenshot of mbed compiler environment

Click New to create a new program, and enter a filename.  Your newly created program will appear in the Program Workspace on the left of the screen.  If you expand the program by clicking on the +, you will see something called main.cpp - double click on this, it will open your program in the compiler environment. As you can see, any new program already has some lines of code, shown in Figure 7.

To get the program onto the mbed, you first have to compile it, by clicking the "Compile" button in the toolbar.  This will create the binary code, which will ultimately be downloaded to the mbed.  If the compiling goes well,

Figure 7 –The default code included in any new mbed program

you will get a "Success!" message in the compiler output, and a popup will prompt you to download the compiled .bin file to the mbed, as described below.

Whilst the program is downloading, the status LED on the mbed will flash. Once this has stopped, press the Reset Button on the mbed to start your program running. You should see User LED flashing on and off every 0.2 seconds.

- The DigitalOut Component

As you can see, it is very easy to program the mbed.  This is because of the Application Programming Interface (API) that was mentioned previously.   Basically, this is a set of programming building blocks, which are C++ utilities, which allow rapid code development. You can find links to the various library blocks from the mbed website handbook page

http://mbed.org/handbook/Homepage.

You can follow links for each library function that describes the programming syntax as well as giving some examples to make things more obvious.

Based on this, we can explore each line of code in Figure 7.

Please pay attention to the input method. You should use “” not “”, ; not ；, () not （）.

| #include "mbed.h" | This is a header that needs to be at the start of each mbed. |
| --- | --- |
| DigitalOut myled(LED1); | Essentially, what you are doing here is using the DigitalOut component to set LED1 (LD3 on board) as a digital output, and to give that "port" a name, myled. You can do something similar for each of other Digital IO pins, using the format: DigitalOut variablename (pin number) For example, DigitalOut greenled (D5) would set digital-pin 5 as a digital output, with the name ‘greenled’. |
| int main() { | The action of any C++ program is contained within its main() function. The function definition, what goes on inside the function, is contained within the curly brackets, starting immediately after main(), and continuing until the last closing curly bracket. |
| while(1) { | Many embedded systems programs contain endless infinite loop, a section of the program that just goes on repeating forever. In other branches of programming, this is bad practice, but for embedded systems, this is quite standard. The endless loop is created using the while keyword; this controls the code within the curly brackets, which follow. Normally, while is used to set up a loop, which repeats if a certain condition is satisfied, but if we write while(1), this will make the loop repeat endlessly. |

The part of the program that actually causes the LED to switch on and off is contained in the 4 lines within the while loop. There are two calls to the library function ThisThread::sleep_for(), and two statements in which the value of myled is changed.

| myled=1; | means that the variable myled is set to the value 1, no matter what its previous value was. In C++, this is different to "equals", which is represented by "= =". "=" has the meaning "is set to". This sets a logic 1, or voltage of 3.3V on the pin to which LED1 is connected. This will case the LED to light up. |
| --- | --- |
| ThisThread::sleep_for(200ms); | function is from the mbed library - you can find out more about it on the mbed website. The 200 parameter is in milliseconds, and defines the delay length caused by this function. So, the LED will be lit for 200ms |
| myled=0; | means that the variable myled is set to the value 0, whatever its previous value was. This sets a logic 0, or voltage of 0 V on the pin to which LED1 is connected. This will cause the LED to switch off. Note that this will not always turn an LED off; it depends on how the LED is wired in the circuit. |
| ThisThread::sleep_for(200ms); | the LED will be switched off for 200ms |

The program will then loop back to the while(1) statement and the LED flashing will continue forever.

- The DigitalIn Component

The mbed API has a DigitalIn function with a format identical to that of DigitalOut.  Refer to the DigitalIn section of the mbed handbook for further details.

The syntax is DigitalIn variablename (pinnumber);

For example Digitalin switchinput(D5); would set digital-pin 5 as a digital input, with the variable name switchinput.

- Modifying the Program Code

In the main.cpp file, change the value within the ThisThread::sleep_for(); command. Now compile and download the modified code to the mbed, you should now see LED1 (LD3) flashing with a different frequency.

- Mbed Libraries

You can explore the syntax for many of the  mbed libraries by following links from the mbed handbook homepage. http://mbed.org/handbook/Homepage

In this manual, mbed-os 6.17.0 is chosen as the Mbed library and you can click the blue button to select different libraries. Libraries newer than mbed-os 6.15.0 are suggested.

2.  Lab Exercises

Exercise 1 - Switching on and off a pair of LEDs

- Select two LEDs from the parts available in the lab – one should be a red LED and the other should be a green LED.

- Use the breadboard to connect the mbed and the LEDs. Connect the anode of the red LED to digital-pin 5 (D5) on the mbed and the cathode to ground.  The anode of the LED has the longer leg while the cathode has the shorter leg. Connect the anode of the green LED to digital-pin 6 of the mbed board and the cathode to ground.

- Based on what you have learned above, and with reference to the DigitalOut page on the mbed website, generate code that will cause the LEDs to switch on and off, where the red LED is ON while the green LED is off and visa versa.  The duration of illumination of each LED should be 1 second.

- Switch the cathode of the green LED to digital-pin 6 and the anode to ground. Run the program.  What is changed?

- Switch the anode of the green LED back to digital-pin 6.  Connect the cathode to digital-pin 9.

- Modify the program so that digital-pin 9 is set to level of 0. Run your program. While the program is running, measure the voltage at digital-pin 9. Has the sequence of turning ON and OFF the two LEDs changed from step c?

- Modify the program so that digital-pin 9 is set to level of 1. Run your program. While the program is running, measure the voltage at digital-pin 9. Has the sequence of turning on and off the two LEDs changed from step c?

Milestone 1: Show your code and the LEDs switching ON and OFF to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Pick a red LED and a green LED from the parts kit. Plug the Nucleo board into your breadboard and wire up: red LED anode to D5, cathode to GND; green LED anode to D6, cathode to GND. Now write a short program using DigitalOut objects to flip the LEDs back and forth — when red is on, green should be off, and the other way round. Set each state to last for 1 second. Try a quick variation: swap the green LED's cathode to D6 and its anode to GND. Run the same code again. Notice how the LED behavior changes with reversed polarity. Then connect the green LED's anode back to D6 and put its cathode on D9. Set D9 as DigitalOut = 0 in your code, run it, and measure the voltage at D9. Repeat with D9 = 1 and compare what happens to the LED sequence. Wrap up by showing the working setup to a demonstrator (Milestone 1).

Please write your code here. Do not forget to include comments.

#include "mbed.h" #define TIME 1s // main() runs in its own thread in the OS int main() { DigitalOut red(D5); DigitalOut green(D6); DigitalOut D9_output(D9); D9_output = 1; while (true) { red = !red; //the inital value is 0 green = !red; ThisThread::sleep_for(TIME); //keep the current situation and rest for 1s, then the next procissing } //ThisThread:in this process "::": }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The two LEDs take turns: red ON / green OFF, then swap. Each state holds for 1 second as set by ThisThread::sleep_for(1s). Pretty straightforward. When you flip the green LED (cathode to D6, anode to GND), the polarity reverses — now the LED lights up when D6 goes LOW, not HIGH. That's because current only flows when the anode is more positive than the cathode. With the cathode on D9: setting D9=0 makes it act like ground, so the green LED works normally. Setting D9=1 puts both sides at ~3.3V, so no current flows and the LED stays dark. Measured voltages: D9≈0V at logic 0, D9≈3.3V at logic 1.

Exercise 2 – Controllable switching of LEDs / Generating a square wave and observing it on an oscilloscope

Below is some code that flashes one of two LEDs, depending on the state of a 2-way switch.  It gives the opportunity to introduce some additional syntax.

| /*Program to flash 1 of 2 LEDS, depending on the state of a 2 way switch of a 2 way |  |  |  |  |
| --- | --- | --- | --- | --- |
| */ |  |  |  |  |
| #include "mbed.h" |  |  |  |  |
| DigitalOut redled(D3); |  |  |  |  |
| DigitalOut greenled(D4); |  |  |  |  |
| DigitalIn switchinput(D5); |  |  |  |  |
| int main() { |  |  |  |  |
| while(1) { |  |  |  |  |

if(switchinput==1) {	//test value of switch input

//execute following block of code if switch input is 1

greenled = 0;	//green LED is off

redled = 1;	//flash red LED

ThisThread::sleep_for(1s);  //sleep 1s

redled = 0;

ThisThread::sleep_for(1s);

}	//end of if

else {

//execute this block of code if switchinput is 0

redled = 0;	//redled is off

greenled = 1;	//flash green LED

ThisThread::sleep_for(1s);

greenled = 0;

ThisThread::sleep_for(1s);

}	//end of else

}	//end of while(1)

}	//end of main

Comments have been added at the beginning of the code using the /* comments */ syntax, which allows comments to span more than one line. The comments are placed between the /* and */ .  Shorter inline comments can be added by first inserting // then placing comment after

| DigitalIn switchinput(D5); | Sets digital-pin 5 as a digital input and assigns variable switchinput to that pin |
| --- | --- |

The code also introduces the if and else keywords and the equal operator ==. This causes the block of code that follows the if and else keywords to be executed if the specific condition is met. In the case of the code above, the condition is that the variable switchinput is equal to 1. If this condition is satisfied (i.e., digital-pin 5 is connected to a logic 1), then the block of code immediately following the if statement is executed. In this case, the green LED connected to digital-pin 4 would be switched off and the red LED connected to digital-pin 3 would flash on and off. If the condition is not satisfied, then the block of code following the else statement would be executed, in this case the red LED connected to digital-pin 3 would be switched off and the green LED connected to digital-pin 4 would flash on and off.

Write a program to generate a 1 kHz square wave, and observe it on an oscilloscope. You can choose whichever output pin you wish as the source of the square wave.

Milestone 2: Show your code and the output waveform to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Get familiar with the if/else structure by reading through the example code in the lab description. It shows how to read a switch on one pin and use it to decide which LED flashes. The real task here: generate a clean 1 kHz square wave. Pick a digital output pin (D3 works well) and write a loop that toggles it with a 500 us delay — that gives a 1 ms period = 1 kHz. Hook up an oscilloscope to the pin and check that the waveform looks right: square shape, 50% duty, stable frequency. Show the code and the scope trace to a demonstrator (Milestone 2).

Please write your code here. Do not forget to include comments.

#include "mbed.h" #define TIME 500 // main() runs in its own thread in the OS int main() { DigitalOut wave(D3); while (true) { wave = !wave; wait_us(TIME); } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

Got a clean 1 kHz square wave on D3, confirmed on the scope. Period measures exactly 1 ms (500 us high, 500 us low), giving 50% duty. The wait_us() function makes microsecond-level timing easy — way more precise than the millisecond-based sleep functions for kHz frequencies. Rising and falling edges look sharp on the scope, so the DigitalOut switching is working properly.

Exercise 3 - Create a square wave whose frequency depends on the position of a switch

Switches are used in many applications.  One important application is internal to the microcontroller.  There are switches at every pin, which are activated to enable signals to be outputted or to allow input signals to be collected. Another application is to permit a microcontroller to connect and disconnect a high voltage/high current source to another circuit.  The microcontroller itself can’t supply sufficient voltage or current to the circuit but can output sufficient voltage and current to control the switch.

Before you can complete the milestone associated with this program, let’s understand the working of a single pole single throw (SPST) switch that will be used in this exercise. Below is the diagrams and truth table for the SPST switch, 74HC4066, that we will be using here.

74HC4066 Logic Symbol                                   74HC4066 Pin configuration

| Input |  |  | Output |
| --- | --- | --- | --- |
| Vcc | E | Y | Z |
| 0 | x | x | 0 |
| 1 | 0 | x | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Truth Table for 74HC4066 operations

The 74HC4066 is a quad (4) SPST analog switch. Each switch features input pin (Y) and output pin (Z) and an active HIGH enable input (E). For instance, for switch 1, input pin is labelled as 1Y, output pin is labelled as 1Z and the enable pin is labelled as 1E. After connecting Vcc and GND to the power supply, when enable pin E is LOW, the analog switch is turned off. By contrast, when enable pin E is HIGH, the analog switch works as a normal switch. Please refer to the truth table above for all the possible configurations.

Modify Program 2 above so that it will output one of two possible frequencies, 200 Hz and 500 Hz depending on the position of a switch (ON or OFF), which you will wire to one of the input pins on your mbed.

Milestone 3: Show your code and that you can change the square wave frequency by using a switch to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

First up, have a look at the 74HC4066 datasheet — it's a quad SPST analog switch. Each switch has an enable pin (E), an input (Y), and an output (Z). When E is HIGH the switch conducts; LOW and it's off. Wire the 4066 into your breadboard: connect Vcc and GND, then hook D3 to the enable pin (1E, pin 13) and D5 to a physical switch for selecting the frequency. The program should read the switch on D5 and output one of two frequencies on D3: 200 Hz (2500 us half-period) when the switch is HIGH, 500 Hz (1000 us half-period) when LOW. Verify both on the scope. Demonstrate the frequency switching to a demonstrator (Milestone 3).

Please write your code here. Do not forget to include comments.

#include "mbed.h" // D3 connected to 74HC4066 pin 13 (1E enable) DigitalOut switch_control(D3); // D5 connected to a physical switch for frequency selection DigitalIn freq_selector(D5); int main() { while(1) { if(freq_selector == 1) { // 200Hz -> period 5ms (5000us) -> 2500us high, 2500us low switch_control = 1; wait_us(2500); switch_control = 0; wait_us(2500); } else { // 500Hz -> period 2ms (2000us) -> 1000us high, 1000us low switch_control = 1; wait_us(1000); switch_control = 0; wait_us(1000); } } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The switch on D5 controls which frequency comes out: 200 Hz (2.5 ms half-period) when the switch reads HIGH, and 500 Hz (1.0 ms half-period) when LOW. The 74HC4066 analog switch does its job — when the enable pin goes HIGH the switch closes and passes the signal through cleanly. LOW and it blocks the signal. The frequency change happens immediately when you flip the switch; no lag or glitching visible on the scope.

Exercise 4 – Multiplexing/Demultiplexing

There are times when more pins are needed to perform tasks than there are pins available. One way to overcome this problem is to use a multiplexer or demultiplexer.  Multiplexers are commonly used as a way to combine many signals into one or a few signals that are then read by the microcontroller.  Demultiplexers allow the microcontroller to send a few signals out to control many circuits.

  - Connect the anodes of 4 LEDs to the outputs of the 2 x 4 demultiplexer.

  - Connect the cathodes of the LEDs to ground.

  - Connect the power and ground to the demultiplexer.

  - Write a program to use the output from 2 pins on the microcontroller to turn ON and OFF the LEDs in a sequence.

  - Consider what would happen if the anodes of the LEDs had been connected to 3.3 V and the cathodes of the LEDs were connected to the output of the multiplexer instead.

Milestone 4: Show your code and explain how to select a specific LED to turn to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

We're using a 2-to-4 demultiplexer (74HC139) here to drive 4 LEDs with just 2 mbed pins. Wire the four LED anodes to the demux outputs Y0 through Y3, and all cathodes to ground. Hook S0 to D5 and S1 to D6 — these are the select lines. Give the chip power and ground. The code is pretty simple: just cycle through the four binary combinations (00, 01, 10, 11) on S0 and S1, holding each state for 1 second. You'll see each LED light up in turn. Think about what changes if the LEDs were connected with anodes to 3.3V and cathodes to the demux outputs — the logic would invert. Show the circuit and explain how to target a specific LED to a demonstrator (Milestone 4).

Please write your code here. Do not forget to include comments.

#include "mbed.h" #define TIME 1s int main() { DigitalOut S1(D5); DigitalOut S0(D6); while (true) { S1 = 0; S0 = 0; ThisThread::sleep_for(TIME); S1 = 0; S0 = 1; ThisThread::sleep_for(TIME); S1 = 1; S0 = 0; ThisThread::sleep_for(TIME); S1 = 1; S0 = 1; ThisThread::sleep_for(TIME); } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The four LEDs light up one at a time in a continuous loop, each staying on for 1 second. The demux select pins follow the pattern: S1S0=00→LED0, 01→LED1, 10→LED2, 11→LED3. Using just 2 output pins to drive 4 LEDs shows why multiplexing is so useful when you're short on I/O. If you'd wired the LEDs with anodes to 3.3V and cathodes to the demux outputs, the whole thing would be active-low instead.

Exercise 5 - Drive a seven-segment display to display 0-9 and a word H E L P repeatedly

In program 1 you switched ON and OFF a pair of LEDs. LEDs are often packaged together to form patterns, digits, alphanumeric characters etc. A number of standard groupings of LEDs are available including a seven-segment display, which is is composed of seven individual LEDs that can be used to display all numerical digits along with quite a few letters from the English alphabet by lighting different combinations of the seven LED segments.

Figure 8a shows the 7 segments labelled A-G. A decimal point (DP) is usually included in the display. The 7 segments display we will use has 10 pins, which are connected as shown in Figure 8b and in datasheet that is on the course website. The LEDs have a common cathode terminal, pins 3 and 8 on the package; the other pins connect to individual segments.  Segment A is connected to pin 7, segment E is connected to pin 1, DP is connected to pin 5, etc.  The cathode will be connected to GND, ground of the mbed.  By applying a sufficiently large positive voltage to the LED, as can be achieved with an mbed DigitalOut command, a given LED can be illuminated.

Figure 8a - Labelling of the 7 segments in   a seven-segment display

Figure 8b - the pin allocation of the Avago segment display

All the segments can be written in a single byte (8 bits), for example in the sequence

(MSB) DP G F E D C B A (LSB)

If you want to represent the digit "0", it would be necessary to illuminate segments A, B, C, D, E, F. Using the above approach, a "0" would be represented by the byte 00111111 (DP and G not lit, the other segments are illuminated). This is represented in hexadecimal notation as 0x3F, where 0x tells us that it is a hexademical notation and 3F is the hex representation for 0011 1111.

The advantage of this approach is that we can connect the 7 segment display to 7 digital output pins of the mbed, using a new mbed API class BusOut, which allows you to group a set of digital outputs together and to write 2 hexadecimal nibbles directly to it. To demonstrate this, tie the breadboard connect pin 3 or 8 of the seven segment display to the Gnd connection of the mbed and the other pins of the seven segment display to mbed pins that can be configured as digital outputs. Do this in a reasonably logical way, such as that outlined in the table below.2 Please note that you will need to remove the short circuit between D2 and GND, or D2 cannot output a proper voltage.

| mbed pin | GND | D2 | D3 | D4 | D5 | D6 | D9 | D10 | D11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Display pin | 3 or 8 | 7 | 6 | 4 | 2 | 1 | 9 | 10 | 5 |
| Display segment | - | A | B | C | D | E | F | G | DP |

- Produce a table in your lab book like that below that shows the hexadecimal number you will write to the 8 mbed pins to generate the display values 0-9.  0 and 1 are provided to get you started.

| Display Value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Segment drive in digital | 0011 1111 | 0000 0110 | 0101 1011 | 0100 1111 | 0110 0110 | 0110 1101 | 0111 1101 | 0000 0111 | 0111 1111 | 0110 1111 |
| Segment drive in hex | 0x3F | 0x06 | 0x5B | 0x4F | 0x66 | 0x6D | 0x7D | 0x07 | 0x7F | 0x6F |

Having worked out which values to write to the mbed pins, the issue of coding this can be considered with reference to the program below.

/* Program to demonstrate driving 7 segment display.	This example shows how to display digits 0,1,2,3 in turn

*/

#include "mbed.h"

BusOut display(D2,D3,D4,D5,D6,D9,D10,D11);	//segments A,B,C,D,E,F,G,DP int main () {

while(1) {

for (int i=0; i<4;i++){

switch(i){

case 0: display = 0x3F; break;	//display 0 case 1: display = 0x06; break; //display 1

case 2: display = 0x5B; break;	//display 2 case 3: display = 0x4F; break;	//display 3

}	//end of switch

ThisThread::sleep_for(500ms);	//display value for 0.5s

|  |  | } | //end | of | for |
| --- | --- | --- | --- | --- | --- |
|  | } |  | //end | of | while |
| } |  |  | //end | of | main |

The new commands in the above program are:

| BusOut | needs to have a name specified - in this case display, and then lists in brackets the pins which will be members of that bus. |
| --- | --- |
| for | This is just a loop. In this example, variable i (an integer (int) in this case) is initially set to 0 (i=0), and on each iteration of the loop it is incremented by 1 (i++), so long as (in this case) i<4 the loop will repeat. When it reaches 4, the loop terminates. |
| switch, case, break | Used together, these words provide a mechanism to permit one item to be chosen from a list. In this case, as the variable i is incremented, its value is used to select the word that is sent to the display so that the correct digit can be illuminated. |

We now have quite a few nested blocks of code. The switch block lies within the for block, which lies inside the while block, which lies inside the main block. To clarify which curly bracket terminates each block, these have been commented in the code section above.

- Based on your table to determine the hexadecimal values required to produce the digital 0-9 on the seven segment display, modify the code above to count from 0-9.

- What happens if the time each number is displayed is reduced by a factor of 50 to 0.01s?

The mbed has an internal clock.  The clock can be used with the while statement to begin or end an operation.

d.	 Modify your code from part (b) to have the seven-segment display flash the letters H E L P in sequence,

  - where each letter is displayed in sequence for 0.2 s.

  - then there is no display for 0.2 s

  - step (i) and (ii) are repeated 5 times and then the program stops

Milestone 5: Show your code and the working display to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Grab a 7-segment display and look up its datasheet — pins 3 and 8 are the common cathode (connect to GND), and the other pins drive segments A through G plus the decimal point. The smart way to drive it: use BusOut to group 7 (or 8) mbed digital pins into a single port. We'll map A0→A, D3→B, D4→C, A2→D, A1→E, D9→F, D10→G, D11→DP. This lets you write a single hex byte to display any digit. First, work out the hex values for digits 0-9 on paper using the segment-to-bit mapping (MSB: DP,G,F,E,D,C,B,A :LSB). Then code it up with a for loop and switch/case block to count 0-9 with 500 ms per digit. Try speeding it up to 10 ms and see what happens. For the second part, write a program that spells out H-E-L-P on the display: each letter for 0.2s, then 0.2s blank. Repeat the whole sequence five times and then stop. Show it working to a demonstrator (Milestone 5).

Please write your code here. Do not forget to include comments.

#include "mbed.h" BusOut display(A0,D3,D4,A2,A1,D9,D10,D11); //segments A,B,C,D,E,F,G,DP int main() { while (true) { for (int i=0; i<10; i++){ switch(i){ case 0: display = 0x3F; break; case 1: display = 0x06; break; case 2: display = 0x5B; break; case 3: display = 0x4F; break; case 4: display = 0x66; break; case 5: display = 0x6D; break; case 6: display = 0x7D; break; case 7: display = 0x07; break; case 8: display = 0x7F; break; case 9: display = 0x6F; break; } ThisThread::sleep_for(500ms); } } } // === Part (d): Display 'HELP' 5 times === #include "mbed.h" // Pin definitions: seg A,B,C,D,E,F,G,DP in that order BusOut display(A0,D3,D4,A2,A1,D9,D10,D11); int main() { // Part (iii): repeat the sequence 5 times for (int i = 0; i < 5; i++) { // Part (i): display H, E, L, P in sequence, 0.2s each display = 0x76; // 'H' ThisThread::sleep_for(200ms); display = 0x79; // 'E' ThisThread::sleep_for(200ms); display = 0x38; // 'L' ThisThread::sleep_for(200ms); display = 0x73; // 'P' ThisThread::sleep_for(200ms); // Part (ii): blank the display for 0.2s display = 0x00; ThisThread::sleep_for(200ms); } // Part (iii): halt the program. // Use an infinite loop to hold the MCU in its final state display = 0x00; // ensure display is blank at end while (true) { ThisThread::sleep_for(1s); // idle, do nothing } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The 7-segment digits 0 through 9 all display correctly with the hex codes I worked out. The BusOut port uses this bit layout (MSB→LSB): DP on D11, G on D10, F on D9, E on D6, D on D5, C on D4, B on D3, A on A0. At 500 ms per digit the counting is easy to read. Drop it to 10 ms and things get interesting — the digits blur together and you start to see persistence-of-vision effects. The individual digits are still distinguishable if you look closely, but the display looks flickery. The HELP sequence works well: each letter appears clearly for 0.2s with a 0.2s gap between them. After five repetitions the display goes dark and stays that way. Segment patterns used: H=0x76, E=0x79, L=0x38, P=0x73.

Exercise 6 - Creating constant output voltages

As you can see from Figure 1, the mbed has can be configured to generate a number of outputs, including an analogue output on analog-pin 4 (A4). The API command for configuring analogue-pin 4 as an analogue output is AnalogOut name (A4), where name is a variable.

We can then assign a value to the variable, Aout=0.25; for example. The value assigned to the variable can be a floating point number between 0.0 and 1.0 which is outputted to analogue-pin 4. The actual output voltage on analogue-pin 4 is between 0 V and 3.3 V so the floating point number between 0.0 and 1.0 is scaled to this range (0.0 will output 0 V, 0.5 will output 1.65 V, 1.0 will output 3.3 V etc).

The code below outputs 2 values of voltage to analogue-pin 4.

/*Program to output 2 voltage levels to analogue-pin 4 Read the output on a multimeter

*/

#include "mbed.h"

AnalogOut Aout(A4);	//create an analogue output on analogue-pin 4

int main() {

while (1) {

Aout=0.33;

ThisThread::sleep_for(2s);

Aout=0.66;

ThisThread::sleep_for(2s);

}

}

- Given the description of the range of analogue voltages that the mbed can output, what are the two output voltages that should appear on pin A4?

- Confirm the two output voltages in the code shown above are as you expect.

- Modify the code to output constant voltages of 0.5 V, 1.0 V, 2.0 V and 2.5 V.

Milestone 6: Show your code and the output voltages to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

The mbed can output real analog voltages on pin A4 using the AnalogOut API. Here's how it works: you write a float between 0.0 and 1.0, and the mbed scales that to a voltage between 0 V and 3.3 V. So Aout = 0.33 roughly gives you 1.09 V. Start by calculating the code values for your target voltages: code = V_target / 3.3. For 0.5V you get ~0.1515, 1.0V → ~0.3030, 2.0V → ~0.6061, 2.5V → ~0.7576. Write the program to cycle through these four values in a loop, holding each for 2 seconds. Grab a multimeter and measure pin A4 against GND to check that the actual voltages match your targets. Show the code and your voltage readings to a demonstrator (Milestone 6).

Please write your code here. Do not forget to include comments.

/* * Lab 6c: Output four constant voltage levels in a loop * Target voltages: 0.5V, 1.0V, 2.0V, 2.5V * Hardware: L432KC development board * Measurement: use a multimeter to measure voltage between A4 and GND */ #include "mbed.h" // Initialize analog output object Aout on analog pin 4 (A4) AnalogOut Aout(A4); int main() { // Mapping: code value (0.0 to 1.0) = target voltage / Vref (3.3V) while (true) { // 1. Output 0.5V (code = 0.5 / 3.3 ≈ 0.1515) Aout = 0.5f / 3.3f; ThisThread::sleep_for(2s); // 2. Output 1.0V (code = 1.0 / 3.3 ≈ 0.3030) Aout = 1.0f / 3.3f; ThisThread::sleep_for(2s); // 3. Output 2.0V (code = 2.0 / 3.3 ≈ 0.6061) Aout = 2.0f / 3.3f; ThisThread::sleep_for(2s); // 4. Output 2.5V (code = 2.5 / 3.3 ≈ 0.7576) Aout = 2.5f / 3.3f; ThisThread::sleep_for(2s); } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

Measured the four voltage levels on A4 with a multimeter. The mapping is linear, as expected: Code 0.1515 (0.5/3.3) → roughly 0.50 V Code 0.3030 (1.0/3.3) → roughly 1.00 V Code 0.6061 (2.0/3.3) → roughly 2.00 V Code 0.7576 (2.5/3.3) → roughly 2.50 V Readings might be off by a few mV depending on the DAC's 12-bit resolution and the tolerance of the 3.3V reference, but they're close enough for most purposes. The relationship between the float value you write and the voltage you get is dead linear.

Exercise 7 - Creating a sawtooth waveform

- Using a for loop, generate a 100 Hz sawtooth waveform with minimum and maximum voltages of 0 V and 3 V, respectively. You will have to work out how many "steps" to have in your sawtooth and then calculate their duration so that the frequency is correct.

- Measure the output signal using an oscilloscope. Determine the frequency of the sawtooth waveform.

- Change the time scale on the oscilloscope so that you can see the individual steps that form the sawtooth.  Are the steps in voltage of uniform magnitude?  Is the increment equal to the step size that you calculated in part a?

Hint:  Read the mbed datasheet to determine the number of bits that the digital-to-analog converter (DAC) has.

Note:  Not all components in a microcontroller (microprocessor) are composed of digital logic gates.  A DAC and its complement, an analogue-to-digital converter (ADC) contain analogue circuits.

Milestone 7: Show your code and the resulting waveform on an oscilloscope to a demonstrator.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Now for something a bit more interesting — create a sawtooth waveform using the DAC on A4. We want a 100 Hz signal ramping from 0V to 3.0V. Here's the plan: break the ramp into 500 discrete steps. At 20 us per step, one full ramp takes 500 × 20 us = 10 ms, which gives us 100 Hz. The voltage increment per step is 3.0V / 500 = 6 mV. When the for loop resets, voltage drops instantly back to zero — that's your sawtooth edge. The L432KC has a 12-bit DAC (4096 levels), so at 3.0V max the hardware resolution is about 0.73 mV — much finer than our software steps. Hook an oscilloscope to A4. You should see a clean linear ramp followed by a sharp drop. Zoom in on the timebase and you can count the individual steps on the rising edge. Show the waveform on the scope to a demonstrator (Milestone 7).

Please write your code here. Do not forget to include comments.

#include "mbed.h" // L432KC DAC1_OUT1 is on pin PA_4 (labeled A4) AnalogOut dac_out(A4); int main() { // --- Experiment parameters --- const int STEPS = 500; // number of steps per cycle const float V_REF = 3.3; // hardware reference voltage const float V_TARGET = 3.0; // max target voltage const float MAX_V_RATIO = V_TARGET / V_REF; // ratio for 3.0V const int STEP_DELAY_US = 20; // 20us per step -> 10ms period -> 100Hz const float increment = MAX_V_RATIO / STEPS; while (true) { // Generate the rising ramp of the sawtooth for (int i = 0; i < STEPS; i++) { // Write current voltage ratio to DAC dac_out.write(increment * i); // Hold this voltage; delay controls overall frequency wait_us(STEP_DELAY_US); } // When loop ends, i resets to 0 and voltage drops instantly to 0V, creating the sawtooth edge } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The sawtooth waveform shows up nicely on the oscilloscope at 100 Hz — about 10 ms per cycle, matching the math (500 steps × 20 us). Each step measures roughly 6 mV (3.0V / 500). The L432KC's 12-bit DAC could do much finer steps (~0.73 mV) if you bumped the step count toward 4096, but 500 steps is plenty to see the staircase shape on the rising edge. The falling edge is basically instant since it's just the for loop resetting. The ramp is linear because we increment the DAC value by a constant amount each iteration. Changing STEP_DELAY_US lets you tune the frequency — shorter delay = faster ramp = higher pitch.

Exercise 8 – Illuminating LEDs depending on analogue input voltage

The real world is analogue, and many sensors have analogue outputs, yet it is essential that a microcontroller have these signals available in a digital form. As shown in Figure 9, the ability to convert from analogue inputs to digital signals is so important that the mbed can handle up to seven analogue inputs at any given time, on analogue-pins A0-A6. The Application Programming Interface for handling analogue input signals is similar in format to many of those we have looked at already in this introduction to the mbed.

A simple way to observe the operation ofw the analogue input is to use a potentiometer to vary the drive voltage being applied to an LED. Figure 9 shows a 10 kpotentiometer connected to an analogue input of the mbed (A6). The analogue input voltage can be used to directly drive the drive analogue output voltage of A4, which is connected to an LED.  The potentiometer is just acting as a   voltage   divider – the   resulting   voltage   will   change   the   brightness   of   the   LED.

Figure 9 – Connection of 10 kpotentiometer and LED to demonstrate operation of analogue in and analogue out functions

/* Program to use analogue output on analogue-pin 4(A4) to control LED brightness via analogue input on analogue-pin 6(A6)

*/

#include "mbed.h"

AnalogOut Aout(A4);	//defines analog output on A4

AnalogIn Ain(A6);	//define analog input on A6

int main() {

while(1) {

Aout=Ain;	//transfer analog in signal to analog out

}

}

- Construct the circuit of Figure 9.

- Implement the code above to vary the brightness of the LED.

- Confirm that you can vary the intensity of the LED using the potentiometer.

- Determine the smallest variation in the trim pot resistance that will cause a change in the intensity of the LED.

Hint:  Read the mbed datasheet to determine the number of bits that the analogue-to-digital (or analog-to-digital) converter (ADC) has.

- What would happen if the trim pot was connected to +1V instead of Vout?  What circuit from Analogue Electronics would you place between the +1V and the trim pot to make the LED intensity per step the same as in step c?

- Connect 4 separate LEDs at 4 digital output pins of the mbed. Write a program that will illuminate the 4 LEDs depending on the value of input voltage that is produced using a potentiometer to divide the output voltage on the Vout pin of the mbed. The LED illumination in response to the input voltage should meet the requirements in the table below.

| Voltage | LED1 | LED2 | LED3 | LED4 |
| --- | --- | --- | --- | --- |
| 0 V< Vin <0.6 V | 0 | 0 | 0 | 0 |
| 0.6 V< Vin <1.2 V | 1 | 0 | 0 | 0 |
| 1.2 V< Vin <1.8 V | 1 | 1 | 0 | 0 |
| 1.8 V< Vin <2.4 V | 1 | 1 | 1 | 0 |
| Vin >2.4V | 1 | 1 | 1 | 1 |

- Using a multimeter, confirm that the appropriate LEDs illuminate for the given input voltages.

Milestone 8: Show your code to a demonstrator and demonstrate that the LED pattern is correct for the input voltages.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Wire a 10k potentiometer as a voltage divider: outer pins to 3.3V and GND, wiper to analog input A6. Connect an LED (with series resistor) to analog output A4. Part 1 is dead simple — just write Aout = Ain; in a loop. Turning the pot directly controls LED brightness. The ADC on the L432KC is 12-bit (4096 levels), so the smallest voltage change it can detect is about 0.8 mV. Something to think about: what if the pot were connected to 1V instead of 3.3V? You'd need a non-inverting op-amp with gain 3.3 between the 1V supply and the pot to keep the same per-step brightness resolution. For part 2, connect four LEDs to D2, D3, D4, D5. Write code that reads the pot voltage and lights the LEDs based on these thresholds: no LEDs below 0.6V, then each additional 0.6V lights one more LED, with all four on above 2.4V. Use a multimeter to double-check the switching points. Show both the simple dimmer and the 4-LED level indicator to a demonstrator (Milestone 8).

Please write your code here. Do not forget to include comments.

#include "mbed.h" AnalogOut Aout(A4); AnalogIn Ain(A6); int main() { while (1) { Aout = Ain; } } // === Part (g): 4 LEDs controlled by potentiometer voltage === #include "mbed.h" // Define input and output pins AnalogIn pot(A6); DigitalOut led1(D2), led2(D3), led3(D4), led4(D5); int main() { while(1) { float val = pot.read(); // read value between 0.0 and 1.0 float voltage = val * 3.3f; // convert to actual voltage for threshold comparison // Determine which LEDs to light based on voltage if (voltage < 0.6f) { led1 = 0; led2 = 0; led3 = 0; led4 = 0; } else if (voltage < 1.2f) { led1 = 1; led2 = 0; led3 = 0; led4 = 0; } else if (voltage < 1.8f) { led1 = 1; led2 = 1; led3 = 0; led4 = 0; } else if (voltage < 2.4f) { led1 = 1; led2 = 1; led3 = 1; led4 = 0; } else { led1 = 1; led2 = 1; led3 = 1; led4 = 1; } ThisThread::sleep_for(50ms); // small delay to avoid excessive polling } }

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The simple Aout=Ain demo works as you'd expect: turn the pot and the LED follows. Smooth brightness control across the whole range. With a 12-bit ADC, the smallest voltage change the mbed can pick up is 3.3V/4096 ≈ 0.8 mV. So you need to turn the pot enough to cross at least one ADC count before you'll notice a brightness change. The 4-LED level indicator works correctly against the voltage thresholds. Checked with a multimeter at each transition point: LEDs switch on within about 50 mV of the target voltages. The 50 ms loop delay keeps the response feeling snappy without hammering the ADC with reads.

Exercise 9 – Communication between MCU boards via SPI link

Communications systems are important for exchanging information. On the MCU board, there are various means of communication. Here we will investigate the SPI communication between two MCU boards: one MCU is set as ‘master’ (the controller issuing commands), and the other is set as the ‘slave’ (following commends). You will work in teams so you could use two MCU boards.

SPI3 Configuration on L432KC (circled by red)

We use the SPI3 configuration output/input on pins D11 D12 and D13. In addition, we need a digital line to select the slave that we wish to talk to, using pin A3. Connect each of these pins directly from one MCU to the same pin on the other. There should be 4 signal wires and a ground connection between the two breadboards. Note that there is also an SPI1 configuration on the L432KC, but not every combination works fine. The test results are shown in the table below, and SPI3 configuration is suggested as it does not lead to confusion.

SPI Configurations test result

Each MCU needs two switches and two LEDs, connected to the appropriate digital I/O pins. The switches from the master are used to control the LEDs on the slave and vice versa.

The following program should be used on the MCU that you have selected to be the master. Please note you need to fill in (?), depending on the pins you use to connect the switches and LEDs.

// Sets the MCU up as Master, and exchanges data with a slave, sending its own switch positions, and displaying those of the slave.

#include "mbed.h"

SPI ser_port(D11, D12, D13); // mosi, miso, sclk

DigitalOut red_led(?); //red led
DigitalOut green_led(?); //green led

DigitalOut cs(A3); //this acts as “slave select”

DigitalIn switch_ip1(?);

DigitalIn switch_ip2(?);

char switch_word;  //word we will send
char recd_val;      //value return from slave

int main() {

while (1){

//Default settings for SPI Master chosen, no need for further configuration

//Set up the word to be sent, by testing switch inputs

switch_word=0xA0; //set up a recognisable output pattern

if (switch_ip1==1)

switch_word=switch_word|0x01; //OR in lsb

if (switch_ip2==1)
    switch_word=switch_word|0x02; //OR in next lsb

cs = 0; //select slave

recd_val=ser_port.write(switch_word); //send switch_word and receive data

cs = 1;

ThisThread::sleep_for(10ms);

//set leds according to incoming word from slave

red_led=0;		//preset both to 0
green_led=0;

recd_val=recd_val&0x03;  //AND out unwanted bits

if(recd_val==1)

red_led=1;
if(recd_val==2)

green_led=1;
if(recd_val==3){

red_led=1;

green_led=1; }

}

}

The following program should be used on the MCU that you have selected to be the slave. Please note you need to fill in (?), depending on the pins you use to connect the switches and LEDs.

// Sets the MCU up as Slave, and exchanges data with the master, sending its own switch positions, and displaying those of the slave.

#include "mbed.h"

SPISlave ser_port(D11, D12, D13, A3);   // mosi, miso, sclk, ssel

DigitalOut red_led(?); //red led

DigitalOut green_led(?); //green led

DigitalIn switch_ip1(?);

DigitalIn switch_ip2(?);

char switch_word ;	//word we will send
char recd_val; 		//value from master

int main() {

while (1){

//Set up the word to be sent, by testing switch inputs

switch_word=0xA0;		//set up a recognisable output pattern
if(switch_ip1==1)

switch_word=switch_word|0x01; //OR in lsb

if(switch_ip2==1)

switch_word=switch_word|0x02; //OR in next lsb

if(ser_port.receive()) { //test if data has been transmitted

recd_val = ser_port.read(); //read byte from master      ser_port.reply(switch_word); //make this the next reply

}

//set leds according to incoming word from slave

red_led=0;		//preset both to 0
green_led=0;

recd_val=recd_val&0x03; //AND out unwanted bits

if(recd_val==1)

red_led=1;
if(recd_val==2)

green_led=1;
if (recd_val==3){

red_led=1;

green_led=1; }

}

}

- Finish the two programs

- Download the programs on the appropriate boards

- Set up the boards, switches and LEDs

- Run the programs and have a look at the data and clock signals on the oscilloscope.

Note: For this exercise 1) Do not use wait() and wait_ms() any more, please use ThisThread::sleep_for(), wait_us(), or wait_ns() instead; 2) Connect the GND of two boards together; 3) You may find that LEDs are flickering, while they should be OFF. It may be caused due to various reasons. If you connect the switch input port (on the other board) directly to GND, the LEDs will be OFF; or you connect the switch input port directly to VCC, and the LEDs will be ON.

Milestone 9: Show your working SPI link to a demonstrator and explain the data and clock signals on the oscilloscope.

Procedure: Please briefly describe the exercise procedure. Also provide schematic diagram, output, figure and/or table, if any.

Team up with another lab group — you'll need two MCU boards for this one. We're setting up SPI communication: one board as master, the other as slave. Use the SPI3 pin configuration: D11 (MOSI), D12 (MISO), D13 (SCLK), and A3 as chip select. Run four signal wires plus a ground between the two breadboards. Each board gets two switches and two LEDs on whatever digital pins you have free. The master reads its local switches, sends that byte over SPI, and uses the reply from the slave to set its LEDs. The slave does the same thing in reverse. Fill in the actual pin numbers where the template code has (?) markers. When it's running, put an oscilloscope on the clock and data lines. You'll see bursts of activity on SCLK with data on MOSI and MISO whenever the master initiates a transfer. Demonstrate the working SPI link and explain the scope traces to a demonstrator (Milestone 9).

Please write your code here. Do not forget to include comments.

// See the SPI Master and Slave code templates in the exercise description above. // Fill in the (?) with the actual pin numbers used for switches and LEDs. // Master: SPI on D11(MOSI), D12(MISO), D13(SCLK); CS on A3 // Slave: SPISlave on D11, D12, D13, A3

Please write your main finding. Also, if there are any questions asked in this milestone, include the response to these questions.

The SPI link between the two boards works. Flick a switch on the master — the corresponding LED lights up on the slave, and vice versa. The data exchange is bidirectional: each board sends its switch status and receives the other's. On the oscilloscope you can see SCLK ticking away during each transfer, with MOSI and MISO carrying data in opposite directions simultaneously. The CS line (A3) drops low at the start of a transaction and goes high when it's done. Between transactions there's ~10 ms of silence on all lines. Having the two boards share a common ground is essential — without it the signal levels float and you get garbage data. The 0xA0 base pattern in switch_word makes the data bytes easy to spot on the scope since the top bits stay constant while the bottom two bits reflect the actual switch states.